# DESCRIPTION / BESKRIVELSE

## TITLE OF THE INVENTION

Method and System for Unilateral Entropy Harvesting from Wireless Channel State Information with Post-Quantum Key Derivation

## INVENTOR(S)

Daniel Mo Houshmand Oslo, Norway

## ASSIGNEE

QDaria AS Oslo, Norway

## CROSS-REFERENCE TO RELATED APPLICATIONS

Related to Norwegian Patent Application No. 20260384 (filed 2026-03-24), “Method and System for Irreversible Data Anonymization Using Quantum Random Number Generation,” which shares the entropy pool infrastructure but claims a distinct invention.

## FIELD OF THE INVENTION

The present invention relates to wireless security, entropy generation, and post-quantum cryptography. More specifically, the invention relates to: (1) a method for extracting general-purpose cryptographic entropy from WiFi Channel State Information (CSI) using a single device without bilateral key agreement; (2) a method for deriving location-locked encryption keys from CSI eigenstructure (Physical Unclonable Environment Keys); and (3) a method for composing CSI entropy with quantum random bytes to derive mesh network keys compatible with NIST FIPS 203 (ML-KEM-768).

## BACKGROUND OF THE INVENTION

### CSI-Based Key Agreement: The State of the Art

Channel State Information (CSI) describes the frequency-domain response of a wireless channel across OFDM subcarriers. Each subcarrier measurement is a complex number encoding amplitude and phase, influenced by multipath propagation, scattering, and the physical environment.

Prior work has exploited CSI for bilateral key agreement, wherein two wireless endpoints simultaneously observe the same channel and extract correlated random bits:

- Mathur et al. (2008), “Radio-Telepathy: Extracting a Secret Key from an Unauthenticated Wireless Channel,” ACM MobiCom. First demonstration of RSS-based bilateral key extraction.

- Jana et al. (2009), ACM MobiCom. Extended bilateral key extraction to CSI (amplitude and phase) across OFDM subcarriers.

- Liu et al. (2012), IEEE TIFS. Adaptive quantization for bilateral CSI key generation with improved bit agreement rates.

- Avrahami et al. (2023). Recent bilateral WiFi-based key exchange with enhanced reconciliation.

Patent literature confirms this bilateral focus:

- WO2007124054A2: Wireless channel-based key agreement requiring both endpoints.

- US20210345102A1: Bilateral CSI key generation with quantization.

- US10402172B1: Symmetric key extraction from shared wireless observations.

- US8015224B1: Channel reciprocity-based key distribution.

### The Unoccupied Gap

All prior art requires two cooperating endpoints that observe the same channel simultaneously. The extracted bits serve exclusively as a shared secret key. No prior system uses CSI measurements from a single device to produce general-purpose entropy bytes suitable for any cryptographic application (key generation, nonce creation, entropy pool seeding, randomness extraction).

This represents a fundamental departure from bilateral key agreement, introducing unilateral entropy harvesting as a distinct application of CSI measurements.

### Physical Unclonable Functions (PUFs) vs. Environment Keys

RF-PUF approaches (e.g., Chatterjee et al. 2018) fingerprint hardware manufacturing variations. The present invention fingerprints the physical RF environment (room geometry, furniture, wall materials) via CSI eigenstructure. The key is bound to a location, not a device.

### Post-Quantum Key Derivation Gap

No prior art combines WiFi CSI entropy with ML-KEM-768 (NIST FIPS 203) key encapsulation for quantum-resistant mesh networking.

## SUMMARY OF THE INVENTION

The present invention provides three interrelated methods:

- Unilateral CSI entropy harvesting: A single device extracts phase LSBs from WiFi CSI subcarrier measurements, applies Von Neumann debiasing, and produces general-purpose entropy bytes. No second endpoint participates.

- Physical Unclonable Environment Key (PUEK): The SVD eigenstructure of complex-valued CSI measurements is captured at enrollment, storing the top-d right singular vectors as a location fingerprint. At key-derivation time, fresh CSI singular vectors are compared via subspace similarity (s=1d∑|⟨vref,i,vnew,i⟩|2) against the enrollment profile. If s meets a configurable threshold (Standard 0.75, Elevated 0.85, High 0.95, Military 0.98), a 32-byte key is derived via HKDF-SHA256. The key is cryptographically bound to the physical location.

- Hybrid CSI+QRNG mesh key derivation: CSI entropy bytes are XOR-combined with quantum random bytes for defense-in-depth. The composed entropy feeds HKDF-SHA256 to derive MeshKey (HMAC-SHA256 beacon authentication) and SipHashKey (SipHash-2-4 frame integrity) compatible with ML-KEM-768 mesh networks.

## DETAILED DESCRIPTION OF THE INVENTION

### System Architecture

#### 1. CSI Capture

The preferred embodiment uses an ESP32-S3 microcontroller configured to capture raw CSI data from WiFi frames. Each CSI frame comprises 56 complex-valued subcarrier measurements conforming to the 802.11n HT20 frame structure (CSI_SUBCARRIERS = 56, defined in crates/zipminator-mesh/src/csi_entropy.rs:22).

#### 2. Phase LSB Extraction

For each complex subcarrier value H_k, the system computes the phase angle via arg(H_k), quantizes it to 256 discrete levels, and extracts the least-significant bit:

quantized = ((phase + PI) / (2 * PI) * 256.0) as u8
lsb = (quantized & 1) != 0

This produces 56 raw bits per CSI frame. Implementation: extract_phase_lsbs() at csi_entropy.rs:96-108.

#### 3. Von Neumann Debiasing

Raw phase LSBs exhibit measurement bias. The VonNeumannExtractor (csi_entropy.rs:36-90) processes consecutive bit pairs:

- (0, 1) -> output bit 0

- (1, 0) -> output bit 1

- (0, 0) or (1, 1) -> discard

This produces approximately 14 unbiased bits per 56-subcarrier frame (50% discard rate typical). Eight accumulated bits form one entropy byte.

#### 4. XOR Defense-in-Depth

The CsiEntropySource (csi_entropy.rs:121-228) optionally accepts a secondary entropy source (e.g., QRNG pool) via with_xor_source() (csi_entropy.rs:143-149). Each debiased byte is XORed with a byte from the secondary source. By the XOR lemma, the composed output has min-entropy at least as high as the stronger individual source.

#### 5. Pool File Writer

The flush_to_file() method (csi_entropy.rs:187-206) appends accumulated entropy bytes to a persistent pool file (csi_entropy_pool.bin) in append mode. This enables the Python CsiPoolProvider (src/zipminator/entropy/csi_pool_provider.py:38-132) to consume CSI entropy from a separate file with full provenance tracking. The CsiPoolProvider raises RuntimeError when the pool is exhausted rather than falling back to os.urandom, preserving provenance integrity.

#### 6. PUEK Enrollment and Verification

Enrollment (puek.rs:121-137):

- Capture complex-valued CSI data across M frames (rows) and K subcarriers (columns), forming matrix C∈CM×K.

- Perform SVD: C=UΣVH, obtaining right singular vectors V (compute_eigenmodes(), puek.rs:86-114).

- Store the top-d right singular vectors Vref=[v1,…,vd] with a similarity threshold from SecurityProfile (puek.rs:35-57): Standard (0.75), Elevated (0.85), High (0.95), Military (0.98), or Custom.

Verification (PuekVerifier::verify_and_derive(), puek.rs:178-217):

- Capture fresh complex-valued CSI data from the same location.

- Compute fresh right singular vectors Vnew via SVD.

- Compute subspace similarity s=1di=1d|⟨vref,i,vnew,i⟩|2 (cosine_similarity(), puek.rs:140-161).

- If similarity >= threshold: derive a 32-byte key via HKDF-SHA256 using enrolled eigenmodes as input keying material, with info string zipminator-puek-v1 (puek.rs:16).

- If similarity < threshold: return EnvironmentMismatch error.

The DerivedKey (puek.rs:229-248) implements ZeroizeOnDrop and redacts its Debug output.

#### 7. Mesh Key Derivation

The EntropyBridge (entropy_bridge.rs:124-219) derives purpose-specific mesh keys from composed entropy:

- MeshKey (16-byte PSK): HKDF-SHA256 with info string zipminator-mesh-psk-v1 (entropy_bridge.rs:23). Used for HMAC-SHA256 beacon authentication (derive_mesh_key(), entropy_bridge.rs:140-145).

- SipHashKey (16-byte): HKDF-SHA256 with info string zipminator-mesh-siphash-v1 (entropy_bridge.rs:26). Used for SipHash-2-4 frame integrity (derive_siphash_key(), entropy_bridge.rs:152-161).

Input keying material requires a minimum of 32 bytes (MIN_ENTROPY_BYTES, entropy_bridge.rs:20). The IKM is zeroized after derivation.

## SECURITY ANALYSIS

### Unilateral vs. Bilateral: Why This Matters

| Property | Bilateral (Prior Art) | Unilateral (This Invention) |
| --- | --- | --- |
| Endpoints required | Two (synchronized) | One |
| Output type | Shared secret key only | General-purpose entropy bytes |
| Reconciliation needed | Yes (bit mismatch correction) | No |
| Uses | Key agreement | Any cryptographic application |
| PQC compatibility | Not addressed | ML-KEM-768 via HKDF |
| Deployment | Both devices must cooperate | Single device, passive capture |

### PUEK vs. RF-PUF

| Property | RF-PUF (Chatterjee 2018) | PUEK (This Invention) |
| --- | --- | --- |
| What is fingerprinted | Hardware manufacturing defects | Physical RF environment |
| Key bound to | Device | Location |
| Key changes when | Device replaced | Room altered |
| Security profiles | Fixed | Configurable (0.75-0.98) |
| Key derivation | Direct from hardware response | HKDF-SHA256 from eigenstructure |

## IMPLEMENTATION

The preferred embodiment is implemented in:

- Rust crate zipminator-mesh: CSI entropy harvester (csi_entropy.rs, 407 lines, 11 tests), PUEK (puek.rs, 393 lines, 11 tests), entropy bridge (entropy_bridge.rs, 368 lines, 11 tests).

- Python module src/zipminator/entropy/csi_pool_provider.py: CSI pool reader with position persistence and file locking (133 lines, 11 tests).

- ESP32-S3: CSI capture platform (802.11n HT20, 56 subcarriers).

Total test coverage: 33 tests across the three Rust modules plus 11 Python tests.

## DRAWINGS

### Figure 1: Single-Device CSI Entropy Harvesting Pipeline

WiFi CSI Frame (56 subcarriers, 802.11n HT20)
  |
  v
+-------------------------------+
|   Phase LSB Extraction        |  extract_phase_lsbs()
|   arg(H_k) -> quantize(256)  |  csi_entropy.rs:96-108
|   -> LSB per subcarrier       |
+---------------+---------------+
                | 56 raw bits per frame
                v
+-------------------------------+
|   Von Neumann Debiasing       |  VonNeumannExtractor
|   (0,1)->0  (1,0)->1         |  csi_entropy.rs:36-90
|   (0,0)->X  (1,1)->X         |
+---------------+---------------+
                | ~14 unbiased bits per frame
                v
+-------------------------------+
|   Byte Accumulator            |  CsiEntropySource
|   8 bits -> 1 entropy byte    |  csi_entropy.rs:121-228
+-------+--------------+-------+
        |              |
        v              v
  +-----------+  +--------------+
  | XOR with  |  | flush_to_file|
  | QRNG pool |  | (append mode)|
  +-----+-----+  +--------------+
        |
        v
+-------------------------------+
|   HKDF-SHA256 Key Derivation  |  EntropyBridge
|   -> MeshKey (16B PSK)        |  entropy_bridge.rs
|   -> SipHashKey (16B)         |
+-------------------------------+

### Figure 2: PUEK Enrollment and Verification

ENROLLMENT                          VERIFICATION
===========                         ============

Complex CSI matrix C                Fresh CSI matrix C'
(M frames x K subcarriers)          (M' frames x K subcarriers)
  |                                   |
  v                                   v
SVD: C = U Sigma V^H               SVD: C' = U' Sigma' V'^H
  |                                   |
  v                                   v
Right singular vectors V            Right singular vectors V'
  |                                   |
  v                                   |
Store top-d vectors V_ref           |
+ threshold (0.75-0.98)            |
  |                                   |
  +------------+----------------------+
               |
               v
      subspace_similarity(V_ref, V_new)
      s = (1/d) * sum |<v_ref_i, v_new_i>|^2
               |
               +--- >= threshold ---> HKDF-SHA256 ---> DerivedKey (32B)
               |
               +--- < threshold ----> EnvironmentMismatch ERROR

### Figure 3: Prior Art Comparison

PRIOR ART (ALL BILATERAL)            THIS INVENTION (UNILATERAL)
=========================            ===========================

Device A <---CSI---> Device B        Device A ---CSI--->  (no Device B)
   |                    |               |
   v                    v               v
 Extract              Extract         Extract entropy
 correlated bits    correlated bits   (general-purpose bytes)
   |                    |               |
   v                    v               v
 Reconcile <---------> Match         Von Neumann debias
   |                                    |
   v                                    v
 Shared Key                           Entropy Pool / PUEK Key / Mesh Keys
 (key agreement only)                 (any cryptographic use)