# CLAIMS / PATENTKRAV

## Independent Claims

Claim 1. A computer-implemented method for unilateral entropy harvesting from wireless channel state information, comprising:

- receiving WiFi Channel State Information (CSI) frames from a wireless interface, each frame comprising complex-valued subcarrier measurements;

- for each subcarrier measurement, computing the phase angle and quantizing said phase to a discrete level;

- extracting the least-significant bit (LSB) of each quantized phase value to produce a raw bit stream;

- applying Von Neumann debiasing to the raw bit stream, wherein consecutive bit pairs are processed such that differing pairs produce an output bit and identical pairs are discarded;

- accumulating debiased output bits into entropy bytes;

wherein the method operates on a single device without requiring a second wireless endpoint, and the output bytes constitute general-purpose entropy suitable for any cryptographic application, distinct from bilateral key agreement protocols that require two cooperating endpoints.

Claim 2. A method for deriving location-locked encryption keys from wireless channel eigenstructure, comprising:

- capturing complex-valued CSI data across multiple frames from a WiFi interface at an enrollment location, forming a matrix C∈CM×K where M is the number of frames and K is the number of subcarriers;

- performing Singular Value Decomposition (SVD) on said CSI matrix to obtain C=UΣVH, where V contains the right singular vectors;

- storing the top-d right singular vectors Vref=[v1,…,vd] as an enrollment profile together with a configurable similarity threshold;

- at key-derivation time, capturing fresh complex-valued CSI data from the same physical location;

- computing fresh right singular vectors Vnew via SVD of the fresh CSI matrix;

- computing subspace similarity as s=1di=1d|⟨vref,i,vnew,i⟩|2;

- if the subspace similarity s meets or exceeds the threshold, deriving a cryptographic key from the enrollment eigenstructure using HKDF-SHA256 with a purpose-specific info string;

- if the subspace similarity s falls below the threshold, rejecting the key derivation request;

wherein the derived key is cryptographically bound to the physical RF environment of the enrollment location and cannot be derived from a different physical location, and wherein the method fingerprints the environment rather than the hardware device.

Claim 3. A method for generating post-quantum mesh encryption keys from composed entropy, comprising:

- obtaining CSI entropy bytes via the method of Claim 1;

- obtaining quantum random bytes from a quantum random number generator (QRNG);

- XOR-combining the CSI entropy bytes with the QRNG bytes to produce composed entropy, wherein the composed output has min-entropy at least as high as the stronger individual source;

- deriving mesh authentication keys using HKDF-SHA256 with the composed entropy as input keying material and distinct purpose-specific info strings for each key type;

wherein the derived keys are suitable for use with ML-KEM-768 (NIST FIPS 203) key encapsulation in a wireless mesh network, and wherein compromise of either individual entropy source does not reduce the security of the composed output below the entropy of the uncompromised source.

## Dependent Claims

Claim 4. The method of Claim 1, wherein the CSI frames comprise 56 complex-valued subcarrier measurements conforming to the IEEE 802.11n HT20 frame structure.

Claim 5. The method of Claim 1, wherein the Von Neumann debiasing processes consecutive bit pairs such that a pair (0, 1) produces output bit 0, a pair (1, 0) produces output bit 1, and pairs (0, 0) and (1, 1) are discarded.

Claim 6. The method of Claim 1, wherein the wireless interface is an ESP32-S3 microcontroller configured to capture raw CSI data from WiFi frames.

Claim 7. The method of Claim 1, further comprising writing the accumulated entropy bytes to a persistent pool file in append mode, enabling offline consumption by a separate process with position tracking.

Claim 8. The method of Claim 2, wherein the configurable similarity threshold is selected from preset security profiles comprising: Standard (0.75, office access and workstations), Elevated (0.85, financial and medical records), High (0.95, government classified and SCIF), Military (0.98, defense, nuclear, and critical infrastructure), or a custom value in the range [0.0, 1.0].

Claim 9. The method of Claim 2, wherein the HKDF-SHA256 key derivation uses a fixed info string for domain separation and an optional salt parameter for network-specific binding.

Claim 10. The method of Claim 3, wherein the XOR combination provides defense-in-depth such that even if the CSI data is fully compromised by an adversary, the composed output retains the full entropy of the QRNG source.

Claim 11. The method of Claim 3, further comprising deriving a SipHash-2-4 frame integrity key from the composed entropy using HKDF-SHA256 with a distinct info string separate from the mesh authentication key.

Claim 12. The method of Claim 3, further comprising deriving a MeshKey for HMAC-SHA256 beacon authentication from the composed entropy using HKDF-SHA256.

Claim 13. The method of Claim 1, wherein a CSI pool provider raises a runtime error when the pool file is exhausted rather than falling back to operating system entropy, thereby preserving the provenance integrity of CSI-derived entropy.

Claim 14. The method of Claim 1, wherein the wireless interface is any IEEE 802.11-compatible device capable of reporting per-subcarrier complex-valued channel state information, including but not limited to 802.11n, 802.11ac, and 802.11ax interfaces with any subcarrier count.